RF-500/550-2 thru 8 Installation Manual

Tools and supplies needed:

- Small blade standard and/or Phillips screwdriver

- Crimping tool

- #36 drill bit

- 3/8 hand drill

- 16 ga. Wire (color coded) if required

- 20 amp inline fuse (Use this option if powering unit directly from battery or a non-fused power supply)

- Crimp-on connectors for attaching RF voltage output wires to the device that is to be switched using wireless.

- 4, #6 self-tapping sheet metal screws to mount receiver.

Carefully read the RF-500/550 manual and installation manual before starting.

The next step would be to determine which devices are to be switched using the RF-500/550. Plan your mounting locating carefully. NWI recommends mounting the receiver unit away from extreme heat. Also choose a location that will make the routing of power in and output’s simplest.

Installation Procedure:

- Once the receiver mounting location is determined, use the receiver mounting flange as a template and mark the mounting holes. Use the #36 drill bit and mount the receiver.

- NWI recommends using a switched 12VDC supply. If the components being switched by the RF-500/550 need to power-up without the ignition switch in the on/accessory position, a latching type switch such as a toggle should be used inline with the power supply. This prevents accidental activation if the transmitter buttons are pressed. Connect power and ground wires to terminals shown in RF-500/550 manual.

- In this step you will need to set the address of the transmitter to the receiver. The step-by-step procedure is explained in the manual. Once programmed the red LED will turn on when a transmitter button is pushed.

- Now its time to hook-up to the components that are to be switched on and off using the RF-500/550. “Do this with the power turned off to the receiver”. Using the manual, determine which button on the transmitter you what to activate the component and see the corresponding terminal (on the standard RF-500 or colored wire on RF-500/550 with color-coded flying leads) on the receiver in the manual. Ground the component to a common ground such as the chassis or battery. Next, run a color-coded 16 ga. wire to the power supply on the component when required. Turn the power on to the base unit. Press the selected transmitter button and 12VDC will be delivered to the component and turn on for as long as the button is depressed.

- Repeat the above steps for each additional components (up to 8) separate functions.

Standard wire color-coding:

  - Button #1 = Yellow

  - Button #2 = White

  - Button #3 = Green

  - Button #4 = Blue

  - Button #5 = Brown

  - Button #6 = Orange

  - Button #7 = Purple

  - Button #8 = Striped

  - Double-pole = Gray

Optional hook-ups:

- When multiple components need to be turned on simultaneously, NWI recommends using a Ford type solenoid for this application. In this situation, bring the power supply for the components into the solenoid using an adequate cable and voltage source. Hook-up the multiple components to the power-out side of the solenoid and use the RF-500/550 to turn the solenoid on to complete the circuit.

- All RF-500/550’s come standard with what is called  “momentary” switching. This means that when the transmitter button is depressed that power is being output from the RF-500/550. Once you stop depressing the button the switch opens and power will stop being sent to the component. NWI can add 1 latching circuit to the RF-500 and up to 4 on the RF-550. With this option, instead of on as long as the button is depressed the component is on, now when the corresponding transmitter button is pressed it with turn-on the component and the component with stay on until the button is pressed to turn it off. An example would be to turn on/off the radio, alarm, etc.  If you have purchased a RF-500 with latching switch capabilities, terminals S1 and when S2 is depressed it will turn off. The RF-550 latching circuits are turned on and off by depressing the same button, with latching available on buttons 1,2,5 and 6.

- The RF-500/550 will have a range of 25 to 75 ft with the standard antenna. To increase, there are 2 options. First is to carefully bend antenna to stick straight up out of the receiver box. This will increase range. Second is to mount the antenna up and with an unobstructed view line, such as in the vehicle window. This will increase the unit range dramatically. Third is to order the optional long-range antenna, which can receive signals up to 1000ft.

Intended usage:

NWI has no control of how the end user intends to hook-up the RF-500/550. Because of this, NWI recommends the these units are intended for “OFF ROAD USE ONLY”

NWI offers no written or expressed liability as to how the units are used.

Copyright, New World Industries Inc., 2006